CdL Groundwater Model — PEST-optimized parameter set (warm-start initial parameters)

These are the parameter values the CdL MODFLOW 6 model is initialised from — its warm start, applied at build time when USE_PEST_PARAMS = True. They are extracted by extract_pest_optimised.py into pest_optimised.npz and are the single best (minimum-phi) realisation of the final iteration of a prior pestpp-ies calibration: realisation 53, iteration 3, phi = 1,798,090 (grid ncpl = 6,609). At build, the Kh multiplier field is applied to the base K, and the boundary-condition multipliers/offset to the GHB, drains and stream inflow.

# 1. Boundary-condition, drain and inflow parameters

Scalar multipliers on the model's base values; the GHB external head is an additive offset in metres.

| Parameter | Represents | Type | Optimised value |
| --- | --- | --- | --- |
| ghb_cond | GHB conductance (eastern alluvial inflow face) | multiplier | × 0.207 |
| ghb_head | GHB external head (eastern face) | additive (m) | + 1.03 m |
| drnseep_cond | Surface-seepage (exfiltration) drain conductance | multiplier | × 0.050 |
| drnwest_u1 | Western underflow drain conductance — layer U1 | multiplier | × 0.052 |
| drnwest_dp | Western underflow drain conductance — deep | multiplier | × 6.805 |
| drnsec_dp | Secondary-outlet drain conductance — deep | multiplier | × 0.050 |
| drnsec_u1 | Secondary-outlet drain — layer U1 (inactive) | multiplier | × 1.000 |
| drnout | Catchment-outlet drain (retired; loader back-compat) | multiplier | × 1.000 |
| sfr_inflow | Stream inlet inflow (fixed forcing; not calibrated) | multiplier | × 1.000 |

# 2. Horizontal hydraulic conductivity (Kh) field

kh_mult is a per-cell multiplier on the conceptual base K (U1 25, U2 40, U3 1.5 m/d); the effective initial K = base × multiplier. Vertical K is tied at Kv = 0.1 × Kh (fixed anisotropy).

| Layer (unit) | Base K (m/d) | Optimised multiplier (min / median / max) | Effective K, m/d (min / median / max) |
| --- | --- | --- | --- |
| U1 (alluvium) | 25 | 0.20 / 1.00 / 3.19 | 5.0 / 25.0 / 79.7 |
| U2 (terraces) | 40 | 0.20 / 1.00 / 4.68 | 8.0 / 40.0 / 187.4 |
| U3 (Plio-Miocene) | 1.5 | 0.20 / 0.22 / 5.00 | 0.30 / 0.34 / 7.5 |

Note: several pilot points reach the 0.2–5 multiplier bounds (U3 in particular sits near its lower bound, median 0.22), reflecting the over-fitting discussed in the strategy note (§7). To refresh this set from a newer calibration, re-run extract_pest_optimised.py against the latest master directory.